Text-		Quantity	Unity	Price	Total
No.     Description		in Euro	in Euro

1		..........	pieces	0,00	...........

Sectional door SW-80, door leaf made of individual, double-skinned galvanized 
sheet steel door sections, building depth of the sections = 80 mm, 
polyurethane (PU) foam core, 100% CFC free. Thermally separated sections. 
Grey white, similar to RAL 9002. Floor seal (x2), header and central seal in EPDM quality. 
Outer surface micro-profiled UNribbed, inner surface stucco design (alternatively outer 
surface ribbed and stucco design). Laterally closed, 2-sided angular frame, 
with screwed-on runner rails and double side seal, made of galvanized steel, 
including lateral guards.
Heat insulation (UD = W/m²K) 0.58 (for installed door 25m² door surface)
UD = 0.27 W/m²k (panel)

"Teckentrup" or equivalent.

Supply and install according to the manufacturer’s instructions.

Ordering dimensions:
Clearance dimensions:
____mm width and
____mm height are = clearance dimensions

Ordering details:
Headroom: ___mm
lateral buffer right ___mm, left ___mm
(for clearance, refer to special equipment)

Dimensions:
Modular dimensions
Width: 2,250 – 10,000 mm
Height: 1,875 – 8,000 mm
(other dimensions on request)

Fittings:
( ) N = Normal fitting
( ) ND = Normal fitting with roof incline
( ) HL = High lift runner rail fitting
( ) HLU = High lift runner rail fitting with bottom torsion spring shaft
( ) HLD = High lift runner rail fitting with roof incline
( ) HLDU = High lift runner rail fitting with roof incline and bottom torsion 
spring shaft
( ) NSH = Low headroom fitting with rear spring shaft 
(not for doors with a direct drive system)
( ) NSD = Low headroom fitting with roof incline
(not for doors with a direct drive system)
( ) VL = Vertical fitting
( ) VLU = Vertical fitting with bottom torsion spring shaft

Door dimensions with door width < 6,000 mm and door height 
< 5,000 mm with a torsion spring shaft
Door dimensions with door width up to 10,000 mm or door height 
up to 8,000 mm with a direct drive system

Required space for doors with torsion springs:
Lateral buffers:
( ) for manual operation min. 110 mm at both sides 
( ) for geared chain at geared chain side min. 185 mm
( ) for shaft drive at drive side min. 210 mm
( ) for chain drive at drive side min. 150 mm

NSH/NSD:
( ) required sideroom 120
( ) required space for geared chain 185
( ) required space for shaft drive 210
( ) required space for chain drive 150

Required space for doors with a direct drive system:
Lateral buffers:
( ) drive side (direct drive system) = 300 mm
( ) bearing side = 150 mm

Headroom for doors with torsion springs:
( ) for N fitting up to door height
Up to door height < 3,600 mm: 400 mm
Door height > 3,601; < 5,500mm: 450 mm
Door height > 5,501 mm: 500 mm

Headroom for doors with a direct drive system:
( ) for N fitting 600 to 700 mm (size dependent)

( ) for NSH fitting min. 310* mm
* 290 mm possible on request
(not available for doors with a direct drive system)

( ) for HL fitting always specify the headroom 
( ) for VL fitting min. headroom = 2 x door height + 500 mm
( ) for direct drive system 2 x door height + 700

Glazing:
( ) sandwich glazing type SW-80
triple glazing, black plastic, rectangular
dimensions: 829 x 407 mm (wxh)

Operation:
Manual operation:
( ) manual chain hoist, sliding bolts, spring break protection
Electrical operation:
( ) shaft drive TAS, 400 V 3ph, 60% ED (power-on time), IP 65, emergency crank handle
( ) deadman control CS 100, IP 65,control voltage 24 V, fully wired, with digital limit switches (AWG) 
( ) pulse control CS 300, IP 65 incl. safety edge mechanism via optical sensor, control voltage 24 V, 
fully wired, with digital limit switches (AWG) 
( ) direct drive system TAR, 400 V 3ph, 60% ED (power-on time), IP 54, emergency crank handle 
( ) deadman control CS 300, IP 65, control voltage 24 V, fully wired, with digital limit switches (AWG)
( ) other controls possible

* ( ) Please tick as appropriate

Manufacturing certificate:
Teckentrup GmbH & Co. KG 
Industriestr. 50 
D-33415 Verl - Sürenheide 
Tel.: +49 (0) 52 46/5 04-0 
Fax: +49 (0) 52 46/5 04-230 
URL: http://www.teckentrup.biz 
Email: info@teckentrup.biz

## Compilation of titles

No.	Title	Total in EUR

Total sum of all titles:	0,00

Sales tax 19,0%:	0,00

Total sumwith sales tax:	0,00

The texts are just specimen texts and can be used as examples. All details and measures have to be checked by the user. The texts make no claim to be complete, the details have to be clarified with the producer of the products.